944 TURBO CUP

Well, some of the mods are minor, but some aren't, and there are a bunch of changes compared to a normal 951. Here's a list from a factory sheet

It also says the car weighs over 400 pounds less than a standard car.

Magnesium racing wheels (8x16F, 9x16R)

Magnesium intake manifold

Magnesium oil pan

928 S4 brakes

Racing brake pads (front pads are 959 part numbers)

Brake cooling kit

Strut tower brace bar over engine

Fiberglass hood with race locks

Exterior power kill switch

Interior power kill switch

Larger higher output turbo unit

Larger sway bars, rear adjustable

High performance engine electronics

Surakrit safety windshield

Safety roll cage

Recaro racing seat

Driver racing harness

Lightweight side mirrors

Special front struts

Special rear shocks with springs

Strengthened rear torsion arms

Heavy duty racing clutch

Quick ratio steering

Higher degree limited slip axle

Strengthened differential and axle gears

Strengthened transmission gears

Close ratio 5th gear (this has been proven incorrect)

Transmission oil cooler

ABS brakes (with switch)

Stiffened main chassis

Strengthened strut towers

Hard rubber suspension mounts

Lightweight battery

Single engine cooling rad

Underbody drag reduction panels

Lightweight noise insulation

Delete - Power steering

Delete - Fender liners

Delete - Radio, speakers

Delete - Headlight washers

Delete - Air conditioning

Delete - Power windows

Delete - Rear wiper

Delete - Engine shield

Delete – Undercoating

Delete – Sunroof

Delete - Power seat

Delete - Leather interior

Delete - Rear trunk release

Delete - Power mirrors

Delete - Door storage pockets

Delete - Cassette storage

For comparison sake:

Here are the phonedial weights for the same sizes (and castings, I would imagine):

8J 52ET

Cast Aluminum: 9.0kg (19.8lb) - part 951.362.116.01

Cast Magnesium: 7.6kg (16.7lb) - part 951.362.116.11

So in switching from aluminum to magnesium, they saved 3.1lb.

However, since the sizes were different (951's run 7" front, and 8" rear, where turbo cup cars ran 8" front and 9" rear) we can compare the savings of rotating mass overall

Aluminum 951 wheels:

F- 16x7- 8.4kg

R- 16x8- 9.0kg

total wheel weight: 34.8kg (76.56lb)

Magnesium 951 cup wheels:

F- 16x8- 7.6kg

R- 16x9- 7.7kg

total wheel weight: 306kg (67.32lb)

TOTAL ROTATIONAL WEIGHT SAVINGS: 9.24lb